TRN-2025-002: Cold Chain Monitoring — Guide for Drivers and Receiving Staff

| Document ID | TRN-2025-002 |
| --- | --- |
| Version | 1.1 |
| Date | 15 September 2025 |
| Author | Supply Chain Operations — Cold Chain Team |
| Audience | Delivery drivers, receiving dock staff, cold chain handlers |

1. Temperature Limits — Know Your Product

| Product Type | Min Temp | Max Temp | If Exceeded |
| --- | --- | --- | --- |
| Fresh produce | 1°C | 5°C | Report if > 8°C for 30+ min |
| Dairy | 2°C | 6°C | Report if > 8°C for 15+ min |
| Frozen goods | -25°C | -18°C | Report ANY thaw above -15°C |
| Pharmaceuticals | 2°C | 8°C | Report ANY breach immediately |

2. Tive IoT Sensors

Most MCG perishable shipments include a Tive IoT temperature sensor. These devices:

- Record temperature every 5 minutes (EU/Americas) or 10 minutes (APAC)

- Transmit data automatically via cellular network

- Flash red LED if temperature limit is exceeded

- DO NOT remove or disable the sensor — it must stay with the shipment

3. Known Sensor Issues

|  | Americas Fahrenheit Issue: Between July and September 2025, some Tive sensors in Americas DCs were configured to display in Fahrenheit without indicating the unit. If your sensor shows '41' it may mean 41°F (= 5°C, which is fine) or 41°C (critical failure). Check the sensor configuration screen. All sensors should now show '°C'. If yours shows no unit, report to IT. |
| --- | --- |

|  | Lagos DC Connectivity: The Lagos DC has daily 6-hour internet blackouts (typically 11:00-17:00 WAT). During this time, Tive sensors continue recording but cannot transmit. Temperature alerts will be delayed. Extra vigilance required during blackout hours — check the sensor LED directly. |
| --- | --- |

4. Receiving Temperature Check

1. Before unloading, check the trailer temperature on the reefer unit display.

2. Use your handheld probe thermometer to spot-check product temperature.

3. Record the temperature in WMS quality module.

4. If ANY product is outside acceptable range: STOP. Do not unload. Call Quality Manager.

5. Pre-Cooling Requirements (Updated October 2025)

After the TransFrio spoilage incident (INC-2025-05102), all carriers must pre-cool transport units to 0°C BEFORE loading. The previous standard was 2°C.

|  | Drivers: Verify your reefer unit shows 0°C or below BEFORE allowing loading to begin. Loading onto a warm trailer is the #1 cause of cold chain failure. |
| --- | --- |

Thomas Brennan

VP Supply Chain

Global Supply Chain Division