MCG-POL-012: Cold Chain Management Policy

| Policy ID | MCG-POL-012 |
| --- | --- |
| Version | 2.3 |
| Effective Date | 01 April 2025 |
| Owner | Thomas Brennan, VP Supply Chain |
| Approved By | Marcus Chen, CIO |
| References | SOP-SC-002, MCG-POL-007, INC-2025-05102, TRN-2025-002 |

1. Purpose

This policy mandates the requirements for maintaining cold chain integrity across MCG's perishable product supply chain, from supplier origin through warehouse storage and last-mile delivery to the customer. Cold chain failures cost MCG over $48M annually in spoilage losses, with the Q3 2025 LATAM corridor alone contributing $2.48M in a single quarter.

2. Temperature Standards

| Product Category | Storage Temp | Transport Temp | Max Exceedance | Sensor Required |
| --- | --- | --- | --- | --- |
| Fresh produce (leafy, berries) | 1-5°C | 1-5°C | 30 min > 8°C | Yes — Tive 5G |
| Root vegetables | 4-10°C | 4-10°C | 60 min > 12°C | Yes — Tive 5G |
| Dairy (milk, cheese, yogurt) | 2-6°C | 2-6°C | 15 min > 8°C | Yes — Tive 5G |
| Frozen goods | -25 to -18°C | -22 to -18°C | ZERO tolerance > -15°C | Yes — Tive 5G |
| Pharmaceuticals | 2-8°C | 2-8°C | ZERO tolerance outside range | Yes — Sensitech |
| Chilled beverages | 3-7°C | 3-7°C | 60 min > 10°C | Yes — Tive 5G |
| Eggs | 5-12°C | 5-12°C | 60 min > 15°C | Regional (EU only) |

3. IoT Monitoring Coverage

| Region | Coverage | Target (FY2026) | Provider | Interval | Known Issues |
| --- | --- | --- | --- | --- | --- |
| EU | 95% | 99% | Tive Solo 5G | 5 min | None significant |
| Americas | 88% | 95% | Tive + Sensitech | 5 min | Fahrenheit data contamination Jul-Sep 2025 (47K readings) |
| APAC | 85% | 95% | Tive Solo 5G | 10 min | Higher intervals due to data costs |
| LATAM | 72% | 90% | Tive Solo 5G | 5 min | Border crossing connectivity gaps |
| Africa | 45% | 75% | Tive Solo 5G | 15 min | Lagos DC: 6-hour daily connectivity blackout |

4. Known Data Quality Issues

|  | AMERICAS FAHRENHEIT CONTAMINATION: Between July and September 2025, Tive sensors in Americas were configured to report in Fahrenheit without a unit indicator. A reading of '41' could mean 41°F (5°C, fine) or 41°C (catastrophic). Approximately 47,000 readings affected. Data remediation in progress. All sensors reconfigured to Celsius as of October 2025. |
| --- | --- |

|  | LAGOS DC CONNECTIVITY: Daily 6-hour internet blackout (typically 11:00-17:00 WAT). Sensors buffer locally and upload when connectivity resumes. Temperature exceedances may go undetected for up to 6 hours. Manual checks every 30 minutes required during blackout. Satellite backup (Starlink) budgeted for Q2 2026. |
| --- | --- |

5. Carrier Requirements

All carriers transporting MCG temperature-sensitive products must:

1. Pre-cool transport units to 0°C BEFORE loading (tightened from 2°C after TransFrio incident). Reefer must show target temp for 15 minutes before doors open.

2. Maintain continuous temperature logging throughout transport.

3. Share real-time telemetry with MCG (API preferred; SFTP daily file minimum).

4. NOT turn off refrigeration during rest stops, refueling, or border waits. Carriers on routes with extended border crossings must have auxiliary power units (APUs).

5. Report any temperature exceedance to MCG within 1 hour of detection.

Reference: INC-2025-05102 (TransFrio LATAM spoilage) — $2.48M losses from carrier refrigeration gap and border crossing delays.

6. Product Disposition Decision

| Exceedance Duration | Product Category | Decision | Authority |
| --- | --- | --- | --- |
| < 15 min, minor (within +2°C) | All | Accept with monitoring | Quality Inspector |
| 15-30 min | Fresh produce, dairy | Quarantine for assessment | Quality Manager |
| 15-30 min | Frozen (no thaw detected) | Accept with documentation | Quality Inspector |
| > 30 min | Fresh produce, dairy | Reject — do not accept delivery | Quality Manager |
| Any thaw above -15°C | Frozen goods | Reject — immediate disposal | Quality Manager |
| Any exceedance | Pharmaceuticals | Reject — return to supplier | Quality Manager + Compliance |

7. Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2022-06-01 | T. Brennan | Initial release |
| 2.0 | 2024-09-01 | T. Brennan | Added IoT coverage targets; pharmaceutical requirements |
| 2.3 | 2025-04-01 | T. Brennan | Post-TransFrio: tightened pre-cooling to 0°C; APU requirement; Lagos blackout documentation |

Thomas Brennan

VP Supply Chain

Global Supply Chain Division